UCS 414: LAB Assignment 6

1. Configuring a Router

a. Initial Configuration of Router

b. Configure Telnet, Consol password, privileged mode password and VTY  lines password, Secure Shell

c. Configure serial interfaces DTE and DCE

Solution:

Introduction

To connect to the router we execute Internetworking Operating System (IOS)  commands. IOS is a software that runs on all routers and allows the user to  manage and configure the processes that occur on the router. IOS is command line interface (CLI) software which accepts user commands and displays router  output.

As a security feature, Cisco IOS provides separate commands into two different  access level modes; user EXEC level and privileged EXEC level.  User EXEC level: allows a person to access only a limited number of basic  monitoring commands. In this mode, the router or switch prompt is displayed  as:

hostname>

The right arrow (>) in the prompt indicates that the router or switch is at the user  EXEC mode.

Privileged EXEC level: allows a person to access all router commands (including  configuration and management) and can be password protected to allow only  authorized people to access the router. In this mode the symbol “# “ is displayed  as the prompt.

hostname#

Router commands

Router > show show running system information

Router > where list active connections

Router > sh? List all commands that begins with “sh”

Press the up arrow to see or reuse the last entered command. Press it again to go  to the command before that. Press the down arrow to go back through the list.

Enter the Privileged EXEC level.

Router > enable [Enter]

Use the Help command ? to see the list of available commands in the privileged  level.

Router # ?

Practice the frequently used privileged commands below:

Router# show ip interface Learn about the status of the interfaces of this  router

Router# show run View the configuration of the router

Router# show running-config View the Running Configuration. (There  are two different configurations stored on the router; the Running Configuration  and the Start‐Up Configuration. The

Running Configuration is your current, working configuration. It is stored in the  RAM memory of the router.)

Router# show startup-config View the Start‐Up configuration. It is the  configuration that

is loaded when the router initializes

Router# show version Find the version and system information about your  router

Router# show memory Find memory information about the router  Router# configure terminal

Router(config)# hostname r1 Change the router name from “Router” to  “r1”. The command prompt will look now r1(config)#

Router(config)# <CTRL>+Z Switch from configuration to privileged mode  In

In the Privileged EXEC level use the show command to see detailed information  about the network interfaces of the router.

Router # show interfaces [Enter]

Now enter the Configuration level to assign an IP address to the router.  Router# configure terminal [Enter]

Use an IP address (eg. 10.128.20.254) and subnet mask 255.255.0.0 for the  values. Then do the following:

Router(config) # interface fa 0/0 [Enter]

This will ensure that you are configuring the 0/0 interface since there could be  multiple interfaces with names 0/0, 0/1 …etc.

Router(config‐if ) # ip address (given ip address) (given subnet mask) [Enter]  e.g. ip address 10.10.10.1/30

Router(config‐if ) #no shutdown [Enter] This will ensure that the ip address  configuration is administratively up. i.e. The ip address is being used for routing  purpose.

Connect the router to the PC using the Crossover cable. Change the IP address  of your computer by doing the following.

Enter the IP address, subnet mask and default gateway (the IP address for the  router) and click ok. Here use the following values:

IP address (of your computer): 10.128.20.1

Subnet mask: 255.255.0.0

Default gateway: 10.128.20.254

If setting for Router use command R1#.-> ip route 0.0.0.0 0.0.0.0 10.128.20.254 To check the connection between your computer and the router, ping the ip  address of the router. Type ping from the PC DOS window and from the router  privileged EXEC login window.

Setting Router Passwords

So far we have been working on a router with no access restrictions enabled. In  the real world, this is not the case. The most basic form of access restriction  would be by the use of passwords. Passwords can be set for console access,  telnet access and privileged EXEC access. Below are the various password  commands on a Cisco router. The commands have to be typed out in privileged  EXEC mode.

Set a console password

Router(config)#line con 0

Router(config-line)#login

Router(config-line)#password cisco

Set a telnet password. This command is also used to enable telnet access for up  to 5 simultaneous logins

Router(config)#line vty 0 4

Router(config-line)#login

Router(config-line)#password cisco

Stop console timing out in cases of inactivity Router(config)#line con 0  Router(config-line)#exec-timeout 0 0

Set the enable password to cisco Router(config)#enable password cisco  Set the enable secret password to peter.

This password overrides the enable password and is encypted within the config  file

Router(config)#enable secret peter